R.S.R. s.r.l. - via Montale, 3 - 20070 Bulgarograsso (CO) - Tel.: 031.3532168 - Fax: 031.891688

UCB-MIND communication

| Code: | RSR641 |
| --- | --- |
| Description | UCB-MIND communication |
| Customer: | VEII |
| File name: | input.doc |

| Revision | Date | Changes | Name |
| --- | --- | --- | --- |
| bozza | 2010/03/15 |  | Ezio |
| Revision | 2010/04/07 | Expanded description and values on status Added credit available on status | Ezio |
| Revision | 2010/10/14 | Added price query command, updated programming message with machine disable and discount | Ezio |
| Draft | 09/05/11 | Added command to know discounted selections | Alessandro |
| Revision | 22/09/12 | Added command LED, DEX, REV and updated command SELL | Alessandro |

1. Channel	2

2. Protocol	2

2.1. Operating commands	2

2.2. Programming commands	3

# Channel

Serial communication:

RxD

TxD

GND

Baud rate: 9600

Parity: none

Data bit: 8

Stop bit: 1

Signal level: TTL

| MIND – CN4 |  | UCB – CN7 |
| --- | --- | --- |
| 4 - GND |  | 2 – GND |
| 2 – RxD |  | 3 – TxD |
| 3 – TxD |  | 4 – RxD |

# Protocol

MIND is the master and the UCB is the slave.

Every command from the master has a reply from the slave.

Basically two types of commands are used:

- Operating commands

- Programming commands

Command/response structure is:

STX	start of message

…	payload

CR/LF	end of message

the “payload” is built by a list of parameters separated by “;” ( semicolon ); sometime the list include optional parametrs, which may not be sent; if a parameter is not sent its “;” is not needed also to be sent. An example is the SELL; command

SELL;110 CR/LF 	will ask to sell selection 110 with the default price

SELL;110;75CR/LF 	will ask to sell selection 110 with the given price of 75C

## Operating commands

The operating commands are a Group of commands to read status and start sales, get machine status and working parameters.

These commands are:

- Read status

Command structure

STX status? CR/LF

Reply structure

STX status;<status_word>;<credit_available> CR/LF

- Make selection

Command structure

STX sell;<selection_row_column>;<optional_sell_price>CR/LF

<selection_row_column> any available machine selection

<optional_sell_price>	  OPTIONAL parameter; if not sent the selling price is the default stored into 				   UCB; if sent, the sell price has this value, even ZERO.

Reply structure

STX status;<status_word>;<credit>;<optional status1>;<optional status2>;<optional status2>;  CR/LF

<selection_row_column>	could be any valid selection for the machine

< status_word >	machine status:

Lower digit

0=ready

1=ready/exact change

2=selection start, motor start moving

3=after (2) when motor stops, selection ok

4=after (2) when motor stops, selection failed

5=error selection locked, nothing happens

6=error selection not available, nothing happens

7=error selection price higher than available credit

8=programming mode

U=unknown message

Higher digit

0x= door close

1x= door open

< credit>	amount available

< optional_status1 >	temperature in refrigerated machine, the string F<temperature>

< optional_status2 >	Health status, the string H<status>

< optional_status3 >	motion sensor status, the string S<status>

- Read price

Command structure

STX price? <selection_row_column>  CR/LF

Reply structure

STX price;<amount>;<discounted>;<status_word>;<credit_available> CR/LF

<selection_row_column>	could be any valid selection for the machine

< amount >	selection price

< discounted >	price to pay (calculated with discounts, if apply)

< status_word >	machine status: see ‘Make selection’ command

< credit_available >	amount available

- Read discounted selection list

Command structure

STX discount? CR/LF

Reply structure

STX discount;<selection_row_column>;<selection_row_column>;.......CR/LF

<selection_row_column>	could be any valid selection for the machine which is discounted

( this data is repeated for any discounted selection, which is limited by a machine parameter  NonVolatileSetup.discountMaxSel  )

- Read list of available selections

Command structure

STX sels? CR/LF

Reply structure

STX sels;<selection_row_column>;<selection_row_column>;.......CR/LF

<selection_row_column>	could be any valid selection for the machine

- Read list of disabled selections

Command structure

STX disabled? CR/LF

Reply structure

STX disabled;<selection_row_column>;<selection_row_column>;.......CR/LF

<selection_row_column>	could be any valid selection for the machine which is disabled

- Read dex data file

Command structure

STX DEX? CR/LF

Reply structure

<full dex file>

- Read software revision of UCB

Command structure

STX REV? CR/LF

Reply structure

STX rev;<machineID>-<firmwarerelease>CR/LF

<machineID>	up to 8 character string, es. VEI-DOOR

<firmwarerelease>	4 character string, es   072W

- Set LED light intenisty

Command structure

STX LED; <LedIntenisty>CR/LF

Reply structure

STX CR/LF

<LedIntenisty>	numeric value 0-100%; led intenisty is set to the programmed value; Motion sensor will not modifiy the programmed intenisty IF the intensity is different from 100%; setting intensity to 100% will enable again motion sensor control of LED light intendity,

- Read the available credit

Command structure

STX Funds? CR/LF

Reply structure

STX funds;<cash>;<cashless> CR/LF

<cash>	available credit inserted as cash ( coins and bills, tokens )

<cashless>	available credit inserted as cashless ( cards, credit cards, etc.. )

- set Escrow function

Command structure

STX escrow CR/LF

Reply structure

STX CR/LF

## Programming commands

The programming commands can be used to upload and download settings on the UCB.

The commands are:

- Read settings

Command structure

STX read? CR/LF

Reply structure

STX <setting1> CR/LF

STX <setting2> CR/LF

…

STX <setting_n> CR/LF

- Store settings

Command structure

STX <setting1> CR/LF

STX <setting2> CR/LF

…

STX <setting_n> CR/LF

Reply structure

STX ok CR/LF

<setting_n> can be any of this

| text file | description |
| --- | --- |
| CSV;1.0.0 machine;AP7000 boot;#0.0.2 firmware;#0.0.o assetID; ddmmyyyy;21/6/2008 10:37 selection;value sel.A1;5 sel.A2;25 ............. sel.A10;5 sel.B1;5 ............. sel.J9;5 sel.J10;5 adv.msg;ACME adv.delay;3 adv.speed;5 group;amount gp.G0;5 gp.G1;10 gp.G2;0 gp.G3;0 gp.G4;0 gp.G5;0 gp.G6;0 gp.G7;0 gp.G8;0 gp.G9;0 group;row;col gs.G0;0;0 gs.G0;0;0 ............. gs.G9;0;0 selGroup;row;col gt.G0;0;0 gt.G0;0;0 ............. gt.G9;0;0 timeframe;group;day;off;on tf.0;0;0;00.00;00.00 tf.0;0;0;00.00;00.00 tf.0;0;0;00.00;00.00 tf.0;0;0;00.00;00.00 ............. tf.0;0;0;00.00;00.00 setting;value dropSensor;0 ds.A1;0 ds.A2;0 ............. ds.J10;0 set.escrow;0 set.forceVend;0 set.fillTube;1 set.percDimming;0 set.timeDimming;0 set.sensitivity;0 set.motorCoupled.A1;0 set.motorCoupled.A3;0 set.motorCoupled.A5;0 ............. set.motorCoupled.J9;0 set.DropHome;1 set.percTax;1050 set.WinValue;156 set.discountFlag;1 set.discountPerc1;5 set.discountDays1;15 set.discountPerc2;20 set.discountDays2;25 set.discountDaysReset;30 td.disable.0;0,1,23,0 td.enable.0;1,2,34,0 td.disable.1;2,3,45,0 td.enable.1;3,4,56,0 td.disable.4;4,10,20,0 td.enable.4;5,11,22,0 td.disable.5;6,5,45,0 td.enable.5;7,6,54,0 td.disable.9;8,11,59,0 td.enable.9;8,12,1,12 selLockout.0;111;122 selLockout.4;121;122;123 selUpfront.0;126;127 selUpfront.1;133;134;135 selUpfront.2;145;146;147;148;149 | - header (must be the first row of the file) - machine type - boot version - firmware version - asset ID settings - date time information was saved - selections price header - selection price for row, column: A is the first row, B the second and so on. All the possible prices for all the machine supported by the UCB are saved, even if not present on the machine type - adv message text - adv message delay - adv message scrool speed - groups price header - price for ‘group 0’ - price for ‘group 1’ … - selections inside the group of price (see before) header - first selection of the price group 0 - second selection of the price group 0 … - last selection of the price group 9 - group of selections header - first selection of the group 0 - second selection of the group 0 … - last selection of the group 9 - timeframes header - timeframe information with the format: tf.g.wd.hh.mm.HH.MM g timeframe number wd week day hh.mm starting time HH.MM ending time - settings header - all drop sensor settings - drop sensor functionality per selection - set bill escrow: 0=no, 1=yes - set force vend: 0=no, 1=yes - set fill tube: 0=no, 1=yes - set LED dimming percentage: 0…100 - set LED dimming time: 0…99 - set camera sensitivity: 0…3 (0=off,3=high sensitivity) - set motor coupling on NAT147: 0=off, 1=on - set automatic motor homing: 0=disable, 1=enable - set tax value with 2 decimals (percentage, eg. 10.50%) - set number of selection for the Winner functionality - set discount programming value - set disable and enable time of up to 10 group: . the 1st number is the group id (0…9) . the 2nd number is the day (0=Monday, 6=Sunday, 7=weekday, 8=everyday) . the 3rd is the hour . the 4th is the minute . the 5th is the AM(=0) or PM (=12) - program list of selections on a lock group (eg. group 0 has selection 111 and 122) - program list of selections on a single group whose supply is managed by the machine (eg. group 0 has selections 126 and 127 alternately dispensed) |

Example of “not yet configured machine” settings

CSV;1.0.0

machine;AP123

boot;#0.0.2

firmware;#8.0.Z

assetID;

ddmmyyyy;4/8/2009 10:52

selection;value

sel.A1;0

sel.A2;0

sel.A3;0

sel.A4;0

sel.A5;0

sel.A6;0

sel.A7;0

sel.A8;0

sel.A9;0

sel.A10;0

sel.B1;0

sel.B2;0

sel.B3;0

…………...

sel.E5;0

sel.E6;0

sel.E7;0

sel.E8;0

sel.E9;0

sel.E10;0

sel.F1;0

sel.F2;0

sel.F3;0

sel.F4;0

sel.F5;0

sel.F6;0

sel.F7;0

sel.F8;0

sel.F9;0

sel.F10;0

sel.G1;0

sel.G2;0

sel.G3;0

sel.G4;0

sel.G5;0

sel.G6;0

sel.G7;0

sel.G8;0

sel.G9;0

sel.G10;0

sel.H1;0

sel.H2;0

sel.H3;0

sel.H4;0

…………...

sel.H8;0

sel.H9;0

sel.H10;0

sel.I1;0

sel.I2;0

sel.I3;0

sel.I4;0

sel.I5;0

sel.I6;0

sel.I7;0

sel.I8;0

sel.I9;0

sel.I10;0

sel.J1;0

sel.J2;0

sel.J3;0

sel.J4;0

sel.J5;0

sel.J6;0

sel.J7;0

sel.J8;0

sel.J9;0

sel.J10;0

adv.msg;

adv.delay;0

adv.speed;0

group;amount

gp.G0;0

gp.G1;0

gp.G2;0

gp.G3;0

gp.G4;0

gp.G5;0

gp.G6;0

gp.G7;0

gp.G8;0

gp.G9;0

group;row;col

gs.G0;0;0

gs.G0;0;0

gs.G0;0;0

gs.G0;0;0

gs.G0;0;0

gs.G0;0;0

…………...

gs.G9;0;0

gs.G9;0;0

gs.G9;0;0

gs.G9;0;0

gs.G9;0;0

selGroup;row;col

gt.G0;0;0

gt.G0;0;0

gt.G0;0;0

gt.G0;0;0

gt.G0;0;0

…………...

gt.G9;0;0

gt.G9;0;0

gt.G9;0;0

gt.G9;0;0

gt.G9;0;0

timeframe;group;day;off;on

tf.0;0;0;00.00;00.00

tf.0;0;0;00.00;00.00

tf.0;0;0;00.00;00.00

tf.0;0;0;00.00;00.00

tf.0;0;0;00.00;00.00

tf.0;0;0;00.00;00.00

tf.0;0;0;00.00;00.00

tf.0;0;0;00.00;00.00

tf.0;0;0;00.00;00.00

tf.0;0;0;00.00;00.00

tf.0;0;0;00.00;00.00

tf.0;0;0;00.00;00.00

tf.0;0;0;00.00;00.00

tf.0;0;0;00.00;00.00

tf.0;0;0;00.00;00.00

tf.0;0;0;00.00;00.00

tf.0;0;0;00.00;00.00

tf.0;0;0;00.00;00.00

tf.0;0;0;00.00;00.00

tf.0;0;0;00.00;00.00

tf.0;0;0;00.00;00.00

tf.0;0;0;00.00;00.00

tf.0;0;0;00.00;00.00

tf.0;0;0;00.00;00.00

tf.0;0;0;00.00;00.00

tf.0;0;0;00.00;00.00

tf.0;0;0;00.00;00.00

tf.0;0;0;00.00;00.00

tf.0;0;0;00.00;00.00

tf.0;0;0;00.00;00.00

tf.0;0;0;00.00;00.00

tf.0;0;0;00.00;00.00

tf.0;0;0;00.00;00.00

tf.0;0;0;00.00;00.00

tf.0;0;0;00.00;00.00

tf.0;0;0;00.00;00.00

tf.0;0;0;00.00;00.00

tf.0;0;0;00.00;00.00

tf.0;0;0;00.00;00.00

tf.0;0;0;00.00;00.00

setting;value

dropSensor;0

ds.A1;0

ds.A2;0

ds.A3;0

ds.A4;0

ds.A5;0

ds.A6;0

ds.A7;0

ds.A8;0

ds.A9;0

ds.A10;0

ds.B1;0

ds.B2;0

ds.B3;0

ds.B4;0

ds.B5;0

ds.B6;0

ds.B7;0

ds.B8;0

ds.B9;0

ds.B10;0

ds.C1;0

ds.C2;0

ds.C3;0

ds.C4;0

ds.C5;0

ds.C6;0

ds.C7;0

ds.C8;0

ds.C9;0

ds.C10;0

ds.D1;0

ds.D2;0

ds.D3;0

ds.D4;0

ds.D5;0

ds.D6;0

ds.D7;0

ds.D8;0

ds.D9;0

ds.D10;0

ds.E1;0

ds.E2;0

ds.E3;0

ds.E4;0

ds.E5;0

ds.E6;0

ds.E7;0

ds.E8;0

ds.E9;0

ds.E10;0

ds.F1;0

ds.F2;0

ds.F3;0

ds.F4;0

ds.F5;0

ds.F6;0

ds.F7;0

ds.F8;0

ds.F9;0

ds.F10;0

ds.G1;0

ds.G2;0

ds.G3;0

ds.G4;0

ds.G5;0

ds.G6;0

ds.G7;0

ds.G8;0

ds.G9;0

ds.G10;0

ds.H1;0

ds.H2;0

ds.H3;0

ds.H4;0

ds.H5;0

ds.H6;0

ds.H7;0

ds.H8;0

ds.H9;0

ds.H10;0

ds.I1;0

ds.I2;0

ds.I3;0

ds.I4;0

ds.I5;0

ds.I6;0

ds.I7;0

ds.I8;0

ds.I9;0

ds.I10;0

ds.J1;0

ds.J2;0

ds.J3;0

ds.J4;0

ds.J5;0

ds.J6;0

ds.J7;0

ds.J8;0

ds.J9;0

ds.J10;0

set.escrow;0

set.forceVend;0

set.fillTube;1

set.percDimming;0

set.timeDimming;0

set.sensitivity;0

set.DropHome;1

set.percTax;2345

set.WinValue;156

set.discountFlag;1

set.discountPerc1;5

set.discountDays1;15

set.discountPerc2;20

set.discountDays2;25

set.discountDaysReset;30

td.disable.0;0,1,23,0

td.enable.0;1,2,34,0

td.disable.1;2,3,45,0

td.enable.1;3,4,56,0

td.disable.4;4,10,20,0

td.enable.4;5,11,22,0

td.disable.5;6,5,45,0

td.enable.5;7,6,54,0

td.disable.9;8,11,59,0

td.enable.9;8,12,1,12

selLockout.0;111;122

selLockout.4;121;122;123

selUpfront.0;126;127

selUpfront.1;133;134;135

selUpfront.2;144

set.motorCoupled.A1;0

set.motorCoupled.A3;0

set.motorCoupled.A5;0

set.motorCoupled.A7;0

set.motorCoupled.A9;0

set.motorCoupled.B1;0

set.motorCoupled.B3;0

set.motorCoupled.B5;0

set.motorCoupled.B7;0

set.motorCoupled.B9;0

set.motorCoupled.C1;0

set.motorCoupled.C3;0

set.motorCoupled.C5;0

set.motorCoupled.C7;0

set.motorCoupled.C9;0

set.motorCoupled.D1;0

set.motorCoupled.D3;0

set.motorCoupled.D5;0

set.motorCoupled.D7;0

set.motorCoupled.D9;0

set.motorCoupled.E1;0

set.motorCoupled.E3;0

set.motorCoupled.E5;0

set.motorCoupled.E7;0

set.motorCoupled.E9;0

set.motorCoupled.F1;0

set.motorCoupled.F3;0

set.motorCoupled.F5;0

set.motorCoupled.F7;0

set.motorCoupled.F9;0

set.motorCoupled.G1;0

set.motorCoupled.G3;0

set.motorCoupled.G5;0

set.motorCoupled.G7;0

set.motorCoupled.G9;0

set.motorCoupled.H1;0

set.motorCoupled.H3;0

set.motorCoupled.H5;0

set.motorCoupled.H7;0

set.motorCoupled.H9;0

set.motorCoupled.I1;0

set.motorCoupled.I3;0

set.motorCoupled.I5;0

set.motorCoupled.I7;0

set.motorCoupled.I9;0

set.motorCoupled.J1;0

set.motorCoupled.J3;0

set.motorCoupled.J5;0

set.motorCoupled.J7;0

set.motorCoupled.J9;0